
[image: image1.png]SToEST

naiats





	Specification
	 
	Other Information
	 

	ELECTRICAL
CURRENT RATING:1.5AMPS    
VOLTAGE RATING:125VAC
INSULATION RESISTANCE:100MEGOHMS MINI
WITHSTAND VOLTAGE: AC 1000V RMS 50Hz or 60Hz 1 MIN
CONTACT RESISTANCE:20MILLMHOS MINI
MECHANICAL
MATING FORCE:2 WIRE LEAD..350g 4 WIRE LEAD..500g 6 WIRE     
                     LEAD..750g 8 WIRE LEAD 900g 
RETENTION STRENGTH:7.7Kg BETWEEN JACK AND PLUG
CRIMP TENSILE STRENGTH:2.27Kg
DURABILITY:200MATING CYCLES MINI  
MATERIAL
HOUSING: STANDARD  ABS  RESIN(UL94V-0)
CONTACT WIRE:0.46mm DIAMETER PHOSPHOR BRONZE    
                     OPTIONAL GOLD PLATING OVER NICKEL
GOLD PLATING:□3u  INCH,□6u  INCH,□15u  INCH,□30u  
                    INCH,□50u  INCH.
WIRE STYLES:UL  1061  26  AWG  STRANDS
	
	


