[image: image1.png]BEEF(PE)ER 2T



[image: image2.png]BEEF(PE)ER 2T



[image: image3.png]TR

13

I

0.8MAX

E B




[image: image4.png]1202
ss
]
Rss

B2




[image: image5.png]


[image: image6.png]


[image: image7.png]Label

ShockProff



[image: image8.png]


[image: image9.png]


[image: image10.png]


[image: image11.png]Label

ShockProff



[image: image12.png]


[image: image13.png]TR

13

I

0.8MAX

E B




[image: image14.png]1202
ss
]
Rss

B2












1. Scope

This specification shall cover the characteristics of the quartz crystal unit C32SJJ412050-24.0000M.
2.
Specification no.:
-
3. Part no.:  C32SJJ412050-24.0000M
4. Electrical specification

	4-1
	Nominal frequency (Fo)
	24.0000 MHz

	4-2
	Holder type
	SMD 3225 (3.2x2.5mm), 4 pads

	4-3
	Mode of oscillation
	AT cut, fundamental

	4-4
	Frequency tolerance (25(C(3(C)
	(30ppm max.

	4-5
	Frequency stability over operating temperature range
	(30ppm max.

	4-6
	Drive level
	100(W max.

	4-7
	Load capacitance
	12pF

	4-8
	Equivalent resistance
	50( max.

	4-9
	Insulation resistance
	500M( min. (D.C. 100V(15V)

	4-10
	Shunt capacitance
	7.0pF max.

	4-11
	Aging (25(C)
	(3ppm max/year

	4-12
	Operating temperature range
	-40(C to +85(C

	4-13
	Storage temperature range
	-45(C to +95(C

	4-14
	Test equipment
	KH1200

	4-15
	Remark
	Marking: laser marking


5.
Dimension, marking, and recommended soldering pattern








Part no.�
:�
C32SJJ412050-24.0000M�
�
Printed on�
:�
� TIME \@ "d-MMM-yy" �30-Sep-16��
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
8�
�






30-Sep-16





Wong





30-Sep-16





Nelson





30-Sep-16





Frankie











X





X





Address	:	Room 1101-3, Mongkok Commercial Centre, 16 Argyle St., 


		Mongkok, Kowloon, Hong Kong SAR, China


Phone		:	(852) 2391-6725, (852) 2391-7306, (852) 2391-6158


Fax		:	(852) 2789-3205, (852) 2789-3349


Homepage	:	� HYPERLINK http://www.asiansources.com/chequers.co ��http://www.chequers-electronic.com�


E-mail	:	� HYPERLINK http://www.asiansources.com/chequers.co ��info@chequers-electronic.com�





APPROVAL SHEET



































CHEQUERS ELECTRONIC (CHINA) LIMITED





�











Manufacturer:�
�
Customer:�
�
Chequers Electronic (China) Limited�
�
�
�
Part no.�
C32SJJ412050-24.0000M�
�
Customer part no.�
-�
�
Part name�
Quartz crystal unit�
�
Spec. no.�
-�
�
Total page


(Includes this page)�
8�
�
Issued date�
30th Sep 2016�
�



	This product is compliant to:


�
RoHS 


Directive 2011/65/EU�
�
�
REACH (168 SVHCs)


Regulation (EC) No. 1907/2006�
�



�
ISO/TS16949:2002�
�
�
�
�



	For customer use


Approved by�
�
�
�



For internal use only


Prepared by�
Checked by�
Approved by�
�
�
�
�
�



	






































Revision�
Description�
Remark�
�
093016/F�
First draft�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�

































	




























































































CHEQUERS ELECTRONIC (CHINA) LIMITED


�











Quartz Crystal Unit


Specification





Part No.: C32SJJ412050-24.0000M





<This product is RoHS and REACH compliant>
























































Part no.�
:�
C32SJJ412050-24.0000M�
�
Printed on�
:�
� TIME \@ "d-MMM-yy" �30-Sep-16��
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
1of 6�
�






Address	:	Room 1101-3, Mongkok Commercial Centre, 16 Argyle St., 


		Mongkok, Kowloon, Hong Kong SAR, China


Phone		:	(852) 2391-6725, (852) 2391-7306, (852) 2391-6158


Fax		:	(852) 2789-3205, (852) 2789-3349


Homepage	:	� HYPERLINK http://www.asiansources.com/chequers.co ��http://www.chequers-electronic.com�


E-mail	:	� HYPERLINK http://www.asiansources.com/chequers.co ��info@chequers-electronic.com�





Part no.�
:�
  C32SJJ412050-24.0000M�
�
Printed on�
:�
� TIME \@ "d-MMM-yy" �30-Sep-16��
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
2 of 6�
�






CQ


24.000





� EMBED PBrush  ���





Unit: mm





Part no.�
:�
  C32SJJ412050-24.0000M�
�
Printed on�
:�
26-Aug-10�
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
3 of 6�
�









Environmental and mechanical characteristics


Item�
Test type�
Condition�
Remark�
�
6-1�
Vibration resistance�
The crystal unit is subject to a vibration (frequency: 0Hz to 500Hz; amplitude: 1.5mm; cycle time: 1 to 2 minutes) for a duration of two hours of each of three mutually perpendicular axes.�
(FL((5ppm


(RS((5( or (10% (whichever is larger)�
�
6-2�
Drop resistance�
The crystal unit is subject to three drops from a height of 100cm on a hard wood floor (thickness: 3cm).�
�
�
6-3�
Shock resistance�
The crystal unit is subject to three shocks (acceleration: 981m/s2; duration of acceleration: 6ms)�
�
�
6-4�
Heat (damped) resistance�
The crystal unit is stored at a temperature of +40(C(2(C with a relative humidity of 90% to 95% for a duration of 96 days.  After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
�
�
6-5�
Low temperature resistance�
The crystal unit is stored at a temperature of -40(C(2(C for a duration of 96 hours.  After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
�
�
6-6�
High temperature resistance 1�
The crystal unit is stored at a temperature of +100(C(2(C for a duration of 96 hours.  After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
�
�
6-7�
High temperature resistance 2�
The crystal unit is stored at a temperature of +155(C(2(C for a duration of 2 hours.  After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
�
�
6-8�
Aging�
The crystal unit is stored at a temperature of +85(C(2(C for a duration of 30 days.  After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
�
�
6-9�
Leakage�
The crystal unit is stored a chamber that is filled with helium (0.6 to 0.65MPa) for a duration of 10 minutes�
(3x10-8mbar.l/s�
�






Part no.�
:�
  C32SJJ412050-24.0000M�
�
Printed on�
:�
26-Aug-10�
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
4 of 6�
�






Item�
Description�
Material�
�
A�
Solder pads�
Gold plating�
�
B�
Package�
Ceramic�
�
C�
Conductive paste�
Epoxy�
�
D�
Electrode�
Ag�
�
E�
Blank�
SiO2�
�






Total cycle time: 200 seconds max.








     120 seconds      10 seconds max.








260(C(5(C











150(C(5(C








Environmental and mechanical characteristics (continue from preceding page)


Item�
Test type�
Condition�
Remark�
�
6-10�
Thermal impact resistance�
The crystal unit is stored at a temperature of -40(C(2(C for 30 minutes.  Then the crystal unit is transferred (transfer time should be less than 30 seconds), and stored at a temperature of +85(C(2(C for 30 minutes.  Such cycle should be repeated 10 times. After that, the crystal unit is placed in standard atmospheric condition for one hour before measurement.�
(FL((5ppm


(RS((5( or (10% (whichever is larger)�
�
6-11�
Resistance to soldering heat�
Reflow soldering (for HC-49S-SMD and SMD crystal unit only)









































�
�
�
6-12�
Solderability�
Terminals of the crystal unit is immersed into a solder bath (235(C(5(C) for a duration of 5 seconds ( 2 seconds�
Solder should cover the immersed surface area no less than 90%�
�









	7.	Material list





Part no.�
:�
  C32SJJ412050-24.0000M�
�
Printed on�
:�
� TIME \@ "d-MMM-yy" �30-Sep-16��
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
5 of 6�
�






� EMBED PBrush  ���





Unit: mm





� EMBED PBrush  ���





� EMBED MSPhotoEd.3  ���





Air-bubbled wrapper sheet





� EMBED PBrush  ���





Packing*


Package�
Dimension


LxWxH (cm)�
Quantity


(pcs)�
Weight


�
�
Quartz crystal unit�
-�
1�
~ 0.2g�
�
Plastic reel�
(18.0�
3000�
~650.0g�
�
Inner box�
18.5 x 18.5 x 3.5�
1 Plastic reel = 3000�
~ 680.0g�
�
Outer carton box�
39.0 x 21.0 x 21.0�
10 Inner boxes = 30000�
~ 7.80kg  �
�
*Please note that the above packing material and dimension given are for reference only.  The actual packing material and dimension may vary depending on the actual quantity to be delivered.





























  























Part no.�
:�
  C32SJJ412050-24.0000M�
�
Printed on�
:�
� TIME \@ "d-MMM-yy" �30-Sep-16��
�
Prepared by�
:�
Frankie�
�
Ver. Ctrl.�
:�
093016/F�
�
Page(s)�
:�
6 of 6�
�









	9.	Note





	9-1	Please contact our sales representatives or engineers before using the products specified in this specification sheet for the following equipment, which require high reliability, and if such equipment fails, which might damage to a third party’s life, body or property.





	(i) Aerospace equipment			(ii) Medical equipment


	(iii) Power plant equipment			(iv) Transportation equipment


	(v) Traffic control equipment			(vi) Disaster control / prevention equipment


	(vii) Undersea navigational equipment	(viii) Data-processing equipment


	(ix) or equipment that requires similar complexity and / or reliability of above equipment





Product specifications in this specification sheet are as of the date that is printed on.  They are 	subject to change or discontinue without prior notice.  Please check with our sales 	representative or engineers for details.





When using our products, please do not exceed the requirements and conditions 	specified in 	this specification sheet.





Should there be any doubt when using our products, please consult our sales representative or 	engineers before using our products.


























  

















	







































































PAGE  

_1305549848

_1305549981

_1214917548

_1305532074.bin

_1214917338

