KLS5-KSD9700

Rated Voltage :AC250V/AC125V; Rated Current:5A~7A Electric intensity:800V Min. Contact resistance:30mΩ Max. Lnsulation resistance: 100MΩ Min. Response speed:≤1℃/min Number of automatic cycles: 6000 Max.(Resistive Load)

Code; Action temperature (°С); Reset temperature; Voltage AC(V); Current (A); Code; Action temperature (°С); Reset temperature; Voltage AC(V); Current (A). 55; 55±5; ≥30±10; 250; 5; 105; 105±5; 80±10; 250; 5. 60; 60±5; 40±10; 250; 5; 110; 110±5; 80±10; 250; 5. 65; 65±5; 45±10; 250; 5; 115; 115±5; 85±10; 250; 5. 70; 70±5; 50±10; 250; 5; 120; 120±5; 90±10; 250; 5. 75; 75±5; 55±10; 250; 5; 125; 125±5; 95±10; 250; 5. 80; 80±5; 55±10; 250; 5; 130; 130±5; 100±15; 250; 5. 85; 85±5; 60±10; 250; 5; 135; 135±5; 105±15; 250; 5. 90; 90±5; 65±10; 250; 5; 140; 140±5; 110±15; 250; 5. 95; 95±5; 70±10; 250; 5; 145; 145±5; 115±15; 250; 5. 100; 100±5; 75±10; 250; 5; 150; 150±5; 120±15; 250; 5. Order Information KLS5-KSD9700-5A80-B X X X (1) (2) (3)(4)(5)(6)(7) 1、Series No. 2、Rated current: 5A 3、Action temperature Code 4、Housing Material: A:Plastic B:Metals 5、Contact form: Unmarked - normally closed K - normally open 6、Wire material: Unmarked - High temperature silicone line P - PVC UL1015 22AWG lead 7、Wire length: Unmarked- 70mm Length 100 - 100mm Length