#### Specifications

- MCU: 168MHz STM32F405RGT6

- IMU: ICM42688-P (SPI)

- Baro: SPL06-001 (I2C)

- OSD: AT7456E (SPI)

- Blackbox: MicroSD slot (SPI)

- 6x UARTs,  1x Softserial_Tx option(INAV/BF)

- 11x PWM outputs (8x Dshot compatible with BF/ArduPilot)

- 1x I2C

- 4x ADC (VBAT, Current, RSSI, Airspeed)

- 1x spare PINIO

- 4x individual ESC power/signal pads

- 1x Group of  G/S1/S2/S3/S4 pads for 4in1 ESC Signal/GND

- Built in inverter on UART2-RX for SBUS input

- Switchable Dual Camera Input

#### PDB Specifications

- Input: 9~36V (3~8S LiPo)

- PDB: 4x 35A (Max.4x 50A)

- BEC: 5V 1.5A

- BEC: 12V 2A

- LDO 3.3V: 200mA

- Current Senor: 220A,  3.3V ADC  (INAV/BF scale 150,  ArduPilot  66.7 A/V)

- Battery Voltage divider 1K:20K (INAV scale 2100,  BF scale 210,  ArduPilot BATT_VOLT_MULT 21.0)

#### Firmware

- ArduPilot: MatekF405-TE

- INAV: MATEKF405TE_SD

- BetaFlight: MATEKF405TE

#### Physical

- Mounting: 30.5 x 30.5mm, Φ4mm with Grommets Φ3mm

- Dimensions:  36 x 46 mm

- Weight: 10g

#### Including

- 1x F405-TE

- 6x silicon grommets M4 to M3

- 1x Rubycon ZLH 35V/470uF (Low ESR)