[image: image1.emf]3mm

(Copper foil )

primary

(Copper foil )

(Copper foil )

3mm

3mm

secondary

TIPE  Polyimide

Bobbin

(Copper foil )

TIPE  Polyimide 3Ts

WIRE   Copper foil 

WIRE   Copper foil 

3mm

2. Winding sequence

(Copper foil )

1. Schematic

TIPE  Polyimide

Transformer construction PSA1600-TC-ETD


[image: image2.emf]Sn60Pb40

2.1. Winding sequence

Bobbin

TUBING  PTFE/TFL/150V

WIRE COPPER  

TO SOLDER


[image: image3.emf]Sn60Pb40

or

3.1.  Core

3.4. TUBING

WIRE  Electrotechnical   Copper foil  (0.2t x 24mm) 

WIRE  Electrotechnical  Copper foil  (0.05t x 24mm) 

ETD-49  20 pins ETD-49  EPCOS B66368-B1020-T1

Primary winding indyctivit  P3-P6  - 600uH. 

3.6.  SOLDER

TUBING    PTFE/TFL/150V

3.5. VARNISH

3.3. TIPE

WIRE Electrotechnical  Copper diamert 1mm

TIPE  Polyimide Tape 0.07t x 33 mm   #92 

or  Polyimide Film  without  Adhesive

High temperature applications to 180 C. 

3.2. WIRE

3.2. Coil former

3. MATERIAL LIST

Varnish  V-1630FS

Ferrits material: №87 EPCOS  (airgap 0.2mm).

To make of cores  B66367-G-X187 +  B66367-G200-X187  for reception of an air gap 0.2mm.

Electrical stenght - 3000VAC, 50 (60) Hz 1 minute / 5mA, from primary  to secondary.

Minimal guaranteed spacing between primary and secondary wire 6mm.

Taiwan Golden Enterprise CO.,LTD.

http://www.twgold.com.tw/index.html

Minnesota mining & mfg.co (3M)

UL NO.E17385

ETD-49 -1 Taiwan Shulin Enterprise Co Ltd.  www.bobbin.com.tw

ETD-49  EPCOS  B66367-G-X187 +  B66367-G200-X187

4. ELECTICAL SPECIFICATION

Underwood indastris of NEW YORK INC

UL NO.E60614

Yoke (ordering code per piece, 2 are required)   B66368-A2000


